CF 60 Circuit Diagram Index

### The circuit diagrams are numbered according to the machine type. The individual numbers (corresponds to the page number) signify the following here:

4-4 00

two-digit number of the circuit diagram

(same for all types)

Machine type

2      CF 80

3      CF 70

4      CF 60

Number of the chapter

(4- Electrics)

### The circuit diagrams that act independently of the field position are presented from circuit diagram number 00 to 14.

### All of the circuit diagrams that are first activated during the threshing by the field position are presented from circuit diagram number 15 to 49.

### The circuit diagrams for combines equipped with touch screen monitor starting with circuit diagram number 50.

### Individual page numbers are reserved for the future and are therefore not assigned.  Every circuit diagram is additionally labeled with a letter.   These letters indicate the machine number that the circuit diagram starts with.  The validity of the circuit diagram is only canceled when a new circuit diagram with the same number and a different letter is substituted for it.  In the process, individual letters in the alphabet can be jumped over (e.g. Circuit Diagram 4-408 A is first substituted with 4-408 D). The letters start according to the alphabet with A.  The individual letters signify the following here:

A		Starting with Machine Number DCF 86 0103 until a subsequent letter is

substituted

Combine harvesters with Design Q (electrical-hydraulic field tracking) are first manufactured starting with Machine Number DCF880031.  There is a legend for every circuit diagram.  The legend is labeled with the same circuit diagram number, but without letters.  If the legend goes on over several pages, they are provided with an additional number (for example 4-300.1) Notes are labeled with a serial number in the circuit diagram, and are then explained in the legend. The circuit diagrams are closed within themselves as a rule, so troubleshooting only has to be carried out on this circuit diagram.  Every circuit diagram starts with the respective fuse.  The wiring in front of the fuse is drawn in the circuit diagram Operating Voltage 4-400.  If there are links to another circuit diagram, for instance if the same fuse is used or if the witch-on conditions are on a different circuit diagram, this is indicated with an arrow and the circuit diagram number.

General Circuit Diagrams (Independent of the Field Position)

## 4-400 A	Operating Voltage Starting with DCF 86 0103

4-401 A	Lighting System Starting with DCF 86 0103

4-402 A	Warning Device Starting with DCF 86 0103

4-403 A	Direction Indicator Starting with DCF 86 0103

4-404 A	Mirror Adjustment Starting with DCF 86 0103

4-405 A	Display Unit Starting with DCF 86 0103

4-408 A	Cab Starting with DCF 86 0103

4-409 A	Air Conditioner Starting with DCF 86 0103

Circuit Diagrams Activated by the Field Position

4-416 A	Threshing Basket Adjustment Design Q Starting with DCF 86 0103

4-417 A	Grain Tank Filling Display Starting with DCF 86 0103

4-419 A	Reel Speed Adjustment Starting with DCF 86 0103

4-420 A	Threshing Drum Speed Adjustment Starting with DCF 86 0103

4-421 A	Fan Adjustment Starting with DCF 86 0103

4-422 A	Threshing Unit and Header Clutch Starting with DCF 86 0103

4-423 A	Unloader Clutch with Unloader Pipe Control Starting with DCF 86 0103

4-424 A	Header Return Starting with DCF 86 0103

4-426 A	Field Tracking Design Q Starting with DCF 86 0103

4-427 A	Chaff-Cutter Starting with DCF 86 0103

4-428 A	Rape Electrical System Starting with DCF 86 0103

4-428 B	Rape Electrical System Starting with DCF 86 0103

4-430 A	Working Hydraulics Design Q Starting with DCF 86 0103

4-431 A	LH AGRO 968 Control System Starting with DCF 86 0103

4-440 A	Electrical Central Lubrication System Starting with DCF 86 0103

4-441 A	All-Wheel Drive Starting with DCF 86 0103

Operating Voltage

Circuit diagrams:  4-400 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Lamps | H 33 | Central processing unit interior light | Can be switched off | Central processing unit |
| Magnets | Y 30 | Stop magnet | For fuel | On the injection pump |
| Motors | M 12 | Compressor seat | 12 volt | In the driver's seat |
|  | M 13 | Starter | 24 volt | On the diesel engine |
|  | M 14 | Motor time counter | 24 volt | On the diesel engine |
|  | M 15 | Auxiliary fuel pump | 24 volt | On the fuel tank |
|  | DLM12V | Dynamo | 12 volt | Exterior on the engine |
|  | DLM24V | Dynamo | 24 volt | Interior on the engine |
| Relays | K 5 | Field / road |  |  |
|  | K 9 | Starting safety | Neutral joystick |  |
|  | K10 | Starting safety | Electronics subassembly, Start Stop |  |
|  | K11 | Starter protection |  |  |
|  | K 12 | On-board computer |  |  |
|  | K 13 | Starting safety | Display Unit 2 |  |
|  | K 27 | Display Unit 2 | Safety circuit |  |
|  | K 58 | Fan delay |  |  |
|  | K 59 |  |  |  |
|  | K 60 | Starting safety |  |  |
|  | K 61 M | Starting safety | Minus controlled |  |
|  | K 61 P | Starting safety | Plus controlled |  |
|  | K A7 | 12-volt on-board voltage | Note page 4-94 | In front of the battery box |
|  | K A 8 | Starter relay |  | Between both dynamos |
| Switches | S 0 | Emergency stop |  | On the gas lever |
|  | S 21 | Swivel discharge pipe |  | In the joystick |
|  | S 29 | Seat switch |  | In the driver's seat |
|  | S 35 | Field / road |  | Cab roof |
|  | S 42 | Steering column switch | Horn | In the steering column |
|  | S 50 | Neutral position |  | Large hydrostatic drive pump |
|  | S 80 | Master battery switch | 4-pole | In front of the battery box |

| Plugs | X 2 |  |  | Central processing unit |
| --- | --- | --- | --- | --- |
|  | X 5 |  |  | Central processing unit |
|  | X 5.1 |  |  |  |
|  | X 6 |  |  | Central processing unit |
|  | X 6.1 |  |  | Under the engine power take-off |
|  | X 7 |  |  | Central processing unit |
|  | X 8 |  |  | Central processing unit |
|  | X 9 |  |  | Central processing unit |
|  | X 12 |  |  |  |
|  | X 13 |  |  | Central processing unit |
|  | X 14 |  |  | Central processing unit |
|  | X 15 |  |  | Central processing unit |
|  | X 24 |  |  |  |
|  | X 25 |  |  |  |
| Plugs | X 26 |  |  |  |
|  | X 90 | Plug connection | Grain yield measurement | Under the seat console |
|  | X A | 12-volt socket | For the hand lamp | In the central processing unit |
|  | X B | 12-volt socket | Cigarette lighter | On the left next to the driver's seat |
|  | X C | 12-volt socket | For the hand lamp | Right lamp unit |
|  | X D | 12-volt socket | For the hand lamp | Left lamp unit |
|  | X E | 12-volt socket | For the cooling box | Under the passenger's seat |
|  | X F | 12-volt socket | For the hand lamp | In front of the battery box |

- Not used (free)

- Also see the circuit diagrams Lighting System 4-401

- Also see the circuit diagrams Cab 4-408

- Also see the circuit diagrams Air Conditioner 4-409

- Also see the circuit diagrams Field Tracking 4-426 and

the circuit diagrams Threshing Basket Adjustment 4-416

- Also see the circuit diagrams Reel Speed Adjustment 4-419 and

Fan Speed Adjustment 4-421

- Also see the circuit diagrams Working Hydraulics 4-430

- Electronically-controlled central lubrication pump (option)

Also see the circuit diagram 4-440

- Also see the circuit diagram Grain Tank Filling Display 4-417

- Radio, also see the circuit diagram Cab 4-408

- See the circuit diagrams Warning Device 4-402 and Display Unit 4-405

- See Presentation F 1 (presented twice) on the right next to F 6

- Alternative connection possibility (close to K 43)

- Subassembly for motor starting in Display Unit 2

- Grounded on Pin 15 when ready to start

Grounded on Pin 10 when there is a dynamo at a standstill and positive when there is a rotating dynamo

- Switch position presented: Emergency stop switch in ready position

- 24 volts applied when the threshing unit clutch is switched in

Also see the circuit diagrams Threshing Unit and Header Clutch 4-423

Lighting System

Circuit diagrams: 4-401 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Lamps | H 9 li | Side lamp | On the left | Corn adapter |
|  | H 9 re | Side lamp | On the right | Corn adapter |
|  | H 10 | Working light |  | In cab roof |
|  | H 10 | Working light |  | In cab roof |
|  | H 11 li | Driving light, left |  | Left lamp unit |
|  | H 11 re | Driving light, right |  | Right lamp unit |
|  | H 12 li | Parking light, left |  | Left lamp unit |
|  | H 12 re | Parking light, right |  | Right lamp unit |
|  | H 15 li | Low beam | Interior left | Cab roof |
|  | H 15 re | Low beam | Interior right | Cab roof |
|  | H 16 | Unloader lighting | Switch-on conditions | Over cab door |
|  | H 17 | Working light |  | On the rail |
|  | H 19 | Back-up light |  | On the outflow hood |
|  | H 22 li | Back light, left |  |  |
|  | H 22 re | Back light, right |  |  |
| Relays | K 1 | Light relay | 12 volt | Central processing unit |
|  | K 3 | Low beam relay | 12 volt | Central processing unit |
|  | K 37 |  |  |  |
|  | K 38 |  |  |  |
|  | K 39 |  |  |  |
| Switches | S 34 | Rotaflare lights |  | In the cab roof |
|  | S 36 | Light switch |  | In the cab roof |
|  | S 42 | Steering column switch | Headlamp flasher | In the steering column |
|  | S 72 | Discharge pipe transport | Limit stop switch | Discharge pipe, center |
| Plugs | X 2 |  |  | Central processing unit |
|  | X 6 |  |  | Central processing unit |
|  | X 6.1 |  |  | Under the engine power take-off |
|  | X 7 |  |  | Central processing unit |
|  | X 8 |  |  | Central processing unit |
|  | X 15 |  |  | Central processing unit |
|  | X 24 |  |  |  |
|  | X 25 |  |  |  |
|  | X 26 |  |  |  |
|  | X 30 |  |  |  |
|  | X 31 |  |  |  |
|  | X 32 |  |  |  |
|  | x 34 |  |  |  |
|  | X H | Trailer socket |  | On the steering axle support |
|  | x S | Header socket |  | Next to the multi-clutch of the hydraulic system |

- Switching position: Discharge pipe moved out (open when discharge pipe is moved in)

- 12 volt on Fuse F 1:E

- Switch lighting (it is only one of all that are drawn in)

- Display unit on the steering column

- For use as a low beam light during road travel with a mounted corn adapter

- 13-pole connector for corn adapter

Warning Device

Circuit diagrams : 4-402 A starting with Machine Number DCF 860103

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Trans-mitters | G 20 | Signal transmitter, backwards |  | On the outflow hood |
|  | G 21 | Horn |  | Under the cab |
| Switches | S 40 | Switch, backwards |  | On the joystick |
| Lamps | H 23 re | Brake light | Right | On the outflow hood |
|  | H 23 li | Brake light | Left | On the outflow hood |
| Plugs | X 2 |  |  | Central processing unit |
|  | X 7 |  |  | Central processing unit |
|  | X 8 |  |  | Central processing unit |
|  | X 9 |  |  | Central processing unit |
|  | X 26 |  |  | Steering column |
|  | X 30 |  |  |  |
|  | X 31 |  |  |  |
|  | X 33 |  |  |  |
|  | X 34 |  |  |  |
|  | X 35 |  |  |  |

- On Relay K 32:85 horn signal for starting release; see Circuit Diagram 400A

- Horn

- Brake light

- Signal backwards

- 24 volt when ignition on

- Required equipment for towing switch on the joystick (brake light actuation when there is a reduction in speed)

- On Relay K 32:30

Direction Indicator

Circuit diagrams: 4-403 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Lamps | H 13 re | Indicator lights | Right front | Driver platform, right |
|  | H 13 li | Indicator lights | Left front | Driver platform, left |
|  | H 21 re | Indicator light | Right back | On the outflow hood |
|  | H 21 li | Indicator light | Left back | On the outflow hood |
| Relays | K 3 | Flasher unit |  | Central processing unit |
| Switches | S 42 | Steering column flasher switch |  | Steering column |
|  | S 43 | Warning light switch |  | Cab roof |
| Plugs | X 2 |  |  | Central processing unit |
|  | X 7.3 |  |  |  |
|  | X 7.4 |  |  |  |
|  | X 8 |  |  | Central processing unit |
|  | X 14 |  |  | Central processing unit |
|  | X 14.1 |  |  |  |
|  | X 15 |  |  | Central processing unit |
|  | X 25 |  |  |  |
|  | X 26 |  |  |  |
|  | X 28 |  |  |  |
|  | X 30 |  |  |  |
|  | X H | Trailer socket |  | Rear axle support |
|  | X S | Header socket |  | Left lamp unit |

1	Not used (free)

2

3	Control sound in the display unit

4	Control lamps in the steering column

5	R	Right flasher

L	Left flasher

6	13-pole header socket for connecting a corn adapter

Mirror Adjustment

Circuit diagrams: 4-404 A starting with Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Motors | M 1 | Mirror adjustment | Right | In the right mirror |
|  | M 2 | Mirror adjustment | Left | In the left mirror |
| Switches | S 58 | Mirror adjustment |  | Control panel |
| Plugs | X 9 |  |  | Central processing unit |
|  | X 10 |  |  | Central processing unit |
|  | X 15 |  |  | Central processing unit |

3	Mirror heating is not connected

Display Unit

Circuit diagrams: 4-405 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Trans-mitter | G-KWT | Cooling water temperature | Analog transmitter | On the engine |
|  | G-TFS | Fuel display | Analog transmitter | On the fuel tank |
|  | G-MÖD | Engine oil pressure | Analog transmitter | On the engine |
| Relays | K KWS | Cooling water level |  | Central processing unit |
| Switches | S 52 | Hand brake |  | Under the platform |
|  | S 70 | Filling level transmitter 3/4 | Grain tank | Grain tank, lower |
|  | S 71 | Filling level transmitter 4/4 | Grain tank | Grain tank, upper |
|  | S 75-1 | Straw area clogging |  | In the outflow hood |
|  | S 75-2 | Straw area clogging | Only for chaff cutter | At the end of the straw area hood |
|  | S Luf/EJ | Ejector control | Switching point 55°C | On the ejector of the air filter |
|  | S LUF | Air filter control | Vacuum switch | On the air filter |
|  | S HÖT | Hydraulic oil temperature | Switching point 85°C | On the hydraulic tank |
|  | S HÖS | Hydraulic oil level |  | On the hydraulic tank |
|  | S KWT | Cooling water temperature | Switching point 103°C | On the engine |
|  | S KWS | Cooling water level |  | On the water cooler |
|  | S MÖD | Engine oil pressure |  | On the engine |
| Plugs | X 2 |  |  | Central processing unit |
|  | X 5 |  |  | Central processing unit |
|  | X 6 |  |  | Central processing unit |
|  | X 14 |  |  | Central processing unit |
|  | X 14.1 |  |  |  |
|  | X 14.2 |  |  |  |
|  | X 15 |  |  | Central processing unit |

1	For the display field starting conditions see the circuit diagrams Operating Voltage 4-400A

2	For the threshing gap and cutting height display, see the circuit diagram Threshing Basket Adjustment Design P

3	For the speed monitoring, see the circuit diagrams Control System  4-431A

4	For the display in the steering column, see the circuit diagrams Lighting System

4-401A

5	Display unit

6	Voltage supply for the display unit

7	Not used (free)

- There are 24 volts applied to K27;87 when the ignition is switched on.  See the circuit diagrams Operating Voltage 4-400A

- On F6 battery side

- On switched F6 side

- See the circuit diagrams Unloader Clutch and Discharge Pipe 4-423A

- Connection 61 from the  24 volt dynamo

- On F 1;4

- Connection 61 from the 12 volt dynamo

Cab

Circuit diagrams : 4-408 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Lamps | H 30 | Control panel lighting | Night (yellow) | Cab ceiling |
|  | H 31re | Cab lighting | Right (white) | Cab ceiling |
|  | H 31li | Cab lighting | Left (white) | Cab ceiling |
| Motors | M 4 | Windscreen wiper motor |  | Cab roof |
|  | M 16 | Windscreen wash pump |  |  |
| Switches | S 30 | Door contact switch |  | Hinge side, top |
|  | S 44 | Windscreen wiper |  | Cab roof |
| Plugs | X E | Cooling box socket |  | Under the passenger seat |
|  | X Br | Loudspeaker on the radio | Brown | hinter Radio |
|  | X Sw | Radio power supply | Black | Behind the radio |
|  | X 9 |  |  | Central processing unit |
|  | X 10 |  |  | Central processing unit |
|  | X 15 |  |  | Central processing unit |

- Also see the circuit diagrams 	 Air Conditioner 4-409A

 Lighting System 4-401A

 Operating Voltage 4-400A

 Mirror Adjustment 4-404A

 Grain Tank Filling Display 4-417A

2	12 volt from Switch S Lighting

3	Interior lighting

4	To Switch S Lighting

5	Loudspeaker, left

6	Loudspeaker, right

7	Connection for mobile radio (located behind the right cover of the radio next to the radio in the cab roof)

8	Radio and mobile radio (pre-equipped)

9	Antenna

- Windscreen wiper

- On parking light

Air Conditioner

Circuit diagrams : 4-409 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Trans-mitters | NT C1 | Temperature sensor | Room air temperature | Between the center of the back wall and the cab roof |
|  | NT C2 | Temperature sensor | Icing protection | In the cab roof |
| Lamps | H 32 | Indicator panel |  |  |
| Motors | M 5 | Evaporator fan |  | Cab roof |
|  | M 6 | Evaporator fan |  |  |
|  | MK | Compressor |  | On the engine |
| Relays | K A1 |  |  | Cab roof, front right |
|  | K A2 |  |  | Cab roof, front right |
|  | K A3 |  |  | Cab roof, front right |
|  | K A4 |  |  | Cab roof, front right |
|  | K A5 |  |  | Cab roof, front right |
| Switches | S 45 | Cab fan |  | Cab roof |
|  | S 31 | Under / overpressure | Safety switch | On the dry filter |
|  | S 32 | Air conditioner switch |  | Cab roof |
|  | S 33 | Potentiometer | Temperature | Cab roof |
| Plugs | X 5 |  |  | Central processing unit |
|  | X 8 |  |  | Central processing unit |
|  | X 9 |  |  | Central processing unit |
|  | X 10 |  |  | Central processing unit |
|  | X 14.2 |  |  |  |
|  | X 16 |  |  | Central processing unit |

- Electronic thermostat

Target value range:		14....26°C ± 1°C

Icing protection:		ν off    =  3°C

ν on  =  7°C

2	Under or overpressure switch combination attached to the dry filter.

3	Not used (free)

4	Harness connector to air conditioner

5	Control lamp in the display unit

6	On the parking light and ground of the cab harness

Threshing Basket Adjustment Design Q (Electrical-Hydraulic Field Tracking)

Circuit diagrams: 4-416 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Trans-mitters | G 30 | Analog transmitter, basket gap |  | On the basket adjustment |
|  | G 31 | Initiator "narrow" | Like the speed initiator | On the basket adjustment |
|  | G 32 | Initiator "wide" |  | On the basket adjustment |
| Motors | M 7 | Actuator motor |  | On the basket adjustment |
| Plugs | X 7 |  |  | Central processing unit |
|  | X 14 |  |  | Central processing unit |
|  | X 14.2 |  |  |  |
|  | X 28 |  |  |  |

1	Card subassembly Basket Adjustment / Field Tracking (with 3 plugs)

2	Down

3	High

4	Data bus

5	Card subassembly Basket Adjustment / Field Tracking Design Q (with 2 plugs)

6	Display unit (6; 7; 8; 9 display )

Grain Tank Filling Display

Circuit diagrams: 4-417 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Lamps | H 25.re | Grain tank lighting | 24 volt ! | In the grain tank, on the right |
|  | H 25.li | Grain tank lighting | 24 volt ! | In the grain tank, on the left |
| Relays | K 19 | Rotaflare light |  | Central processing unit |
| Switches | S 34 | Rotaflare light | For both | Cab roof, front |
|  | S 70 | Filling level transmitter | 3/4 | In the grain tank |
|  | S 71 | Filling level transmitter | 4/4 | In the grain tank |
| Plugs | X 2 |  |  | Central processing unit |
|  | X 15 |  |  | Central processing unit |
|  | X 31 |  |  |  |
|  | X 32 |  |  |  |

1	Control lamps in the display unit

2	Current only with the position "Field"

3	Rotaflare light on the straw area hood

4	Rotaflare light on the cab roof

5	Also see the circuit diagrams Lighting 4-401A

Reel Speed Adjustment

Circuit diagrams: 4-419 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Diodes | D 4 |  | Diode block D 4 | Central processing unit |
| Trans-mitters | G 5 | Driving speed | Initiator | Left rear wheel |
|  | G 17 | Reel speed | Initiator | Reel drive unit on the header |
| Solenoid valve | Y 1 | Proportional valve, reel speed | Battery distributing valve | Under the driver's platform, right |
| Switches | S 7 | Reel speed, automatic system |  | Control panel |
|  | S 8 | Potentiometer, reel speed |  | Control panel |
| Plugs | X 2 |  |  | Central processing unit |
|  | X 7 |  |  | Central processing unit |
|  | X 8 |  |  | Central processing unit |
|  | X 9 |  |  | Central processing unit |
|  | X 10 |  |  | Central processing unit |
|  | X 11 |  |  |  |
|  | X 16 |  |  | Central processing unit |
|  | X S | Plug to the header |  | Under the driver's platform, left |
|  | X 101 | Plug of the computer |  | Cab; right, top |

1	24 volts are applied when the threshing unit clutch has been switched in

2	24 volts are applied when the field position has been activated

3	Other functions when the threshing unit clutch has been switched in

4	FCT = off; starting with Program Version 1.05 for replacement parts

5	Input speed at the LH AGRO 968 on-board computer

6	On F5:7 12 volt (voltage supply initiators, right side)

7	In Diode Block D 4

8	Proportional amplifier, reel and fan speed

Also see the circuit diagram Fan Speed 4-421A

- Pin 1 is presented several times

- There are even more branch-offs on this dashed line

- On Switch S (24 Volt) when the header clutch has been switched in

Also see the circuit diagrams Threshing Unit and Header Clutches 4-422A

12	O 		Manual reel speed (independent of the driving speed)

- Automatic reel speed (dependent on the driving speed)

13	Adjustment values of the proportional amplifier for

"Design P" (mech. field tracking)		"Design Q" (electr. field tracking)

Threshing Drum Speed Adjustment

Circuit diagrams: 4-420 A starting with Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Motors | M 8 | Speed adjustment for threshing drum | Motor with friction clutch | On the lower threshing drum variator |
| Relays | K 17 | Speed adjustment for threshing drum |  | Central processing unit |
| Switches | S 16 | Speed adjustment for threshing drum | 2-pole key | Control panel |
| Plugs | X 2 |  |  | Central processing unit |
|  | X 9 |  |  | Central processing unit |
|  | X 10 |  |  | Central processing unit |
|  | X 16 |  |  | Central processing unit |
|  | X 101 | Computer plug |  | Cab, top right |

1	On Relay K 18 (fan speed adjustment)

2	On the proportional amplifier for fan and reel speed adjustment

3	24 volts are applied when the threshing unit clutch S is switched in

Also see the circuit diagrams Threshing Unit and Header Clutch 4- 422 A

- Activation of the speed adjustment by selection of the speed display "Threshing Drum" in the on-board computer at more than 250 r.p.m. threshing drum speed or switching over in the on-board computer into the chapter Test.

The on-board computer switches ground to the relay K 17.

Fan Speed Adjustment

Circuit diagrams: 4-421 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Solenoid valve | Y 10 | Change in the volume flow |  | Fan adjustment pump |
| Relays | K 18 | Speed adjustment for fan |  | Central processing unit |
|  | K 54 | Drop-off delay | T = 5 seconds | Central processing unit |
| Switches | S 15 | Speed adjustment for fan |  | Control panel |
| Plugs | X 6 |  |  | Central processing unit |
|  | X 9 |  |  | Central processing unit |
|  | X 10 |  |  | Central processing unit |
|  | X 16 |  |  | Central processing unit |
|  | X 101 | Computer plug |  | Cab, top right |

1	Proportional amplifier for reel and fan speed adjustment

2	Also see the circuit diagrams 4-419 A

3	On K 17 (speed adjustment of the threshing drum)

4	24 volts when the field position is activated

5	24 volts when the threshing unit clutch is switched in S 10

6	Activation of the speed adjustment by selection of the speed display "Fan" in the on-board computer at more than 250 r.p.m. threshing drum speed or switching over in the on-board computer into the chapter Test.

The on-board computer switches ground to the relay K 17.

7	The connector for the harness is in the center on harness, on the left-hand side

Threshing Unit and Header Clutches

Circuit diagrams: 4-422 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Diodes | D 3 |  |  | Central processing unit |
| Solenoid valve | Y 11 | Threshing unit clutch |  | On the left side wall, beneath engine |
|  | Y 12 | Header clutch |  |  |
| Relays | K 41 | Safety circuit | Threshing unit | Central processing unit |
|  | K 42 | Safety circuit | Header | Central processing unit |
|  | K 43 | Header rapid stop |  | Central processing unit |
|  | K 44 | Activation of speed monitoring, header |  | Central processing unit |
| Switches | S 10 | Threshing unit clutch |  | Control panel |
|  | S 11 | Header clutch |  | Control panel |
|  | S 20 | Rapid stop key | Header | In the joystick |
|  | S 29 | Seat switch | Pre-equipped | In the driver's seat |
| Plugs | X 6 |  |  | Central processing unit |
|  | X 9 |  |  |  |
|  | X 10 |  |  |  |
|  | X 13 |  |  |  |
|  | X 16 |  |  |  |
|  | X 45 | Card subassembly |  | In the central processing unit |
|  | X 101 | Computer plug for LH Agro 965 |  | In the front right cab capping piece |

- Switch-on condition "Threshing Unit ON" for header return.  Also see the circuit diagram 4-424 A

- Switch-on condition "Header OFF" for header return

Also see the circuit diagram 4-424 A

- Activation of the speed monitoring "Upper Shaft Spindle"

Also see the circuit diagrams Control System 4-431 A

- 24 volts with a switched-in threshing unit S

for a)chaff cutter, also see the circuit diagrams Chaff Cutter 4-427 A

b) speed adjustment of the threshing drum, see the circuit diagram 4-420 A

- 24 volts with a switched-in header S

for a) reel speed adjustment, also see the circuit diagrams 4-419 A

b) electric rape, also see with regard to this the circuit diagrams 4-428 A

c) field tracking Design P, also see the circuit diagrams 4-425 A

field tracking Design Q, also see the circuit diagrams 4-426 A

6	24 volts with activated "Field Position"

7	See the unloader clutch circuit diagrams 4-423 A

- See the chaff cutter circuit diagrams 4-427 A

- Alternative connection possibility (Line 279 ends directly at K 43).  If Line 279 is connected directly to K 43:87a instead of the ground currently existing,

the corresponding safety requirements can be realized by shutting off the header, with a free driver's seat.

10	The connector for the harness is in the center on harness, on the left-hand side

Unloader Clutch with Discharge Pipe Control

Circuit diagrams: 4-423 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Diodes | D 1 |  |  | Central processing unit |
|  | D 2 |  |  | Central processing unit |
| Trans-mitters | G 40 | Safety circuit | For unloading | Grain tank flap |
|  | G 41 | Safety circuit | For unloading | Grain tank flap |
| Lamps | H 29 | Unloading lighting |  | Cab roof under driver's door |
| Relays | K 4 | Unloader clutch | Incremental relay | Central processing unit |
|  | K 30 | Unloader pipe out |  | Central processing unit |
|  | K 31 | Unloader pipe in |  | Central processing unit |
| Switches | S 21 | Swivel discharge pipe | Key | In the joystick |
|  | S 22 | Unloader clutch |  | In the joystick |
|  | S 73 | Unloader pipe moved out |  | On the swivel device |
| Plugs | X 2 |  |  |  |
|  | X 6 |  |  |  |
|  | X 7 |  |  |  |
|  | X 9 |  |  |  |
|  | X 11 |  |  |  |
|  | X 12 |  |  |  |
|  | X 13 |  |  |  |
|  | X 14 |  |  |  |
|  | X 15 |  |  |  |
|  | X 32 |  |  |  |

- Switching position is discharge pipe moved out; the end position has not yet been reached

(opens in the end position of discharge pipe out)

2	The connector for the harness is in the center on harness, on the left-hand side

3	24 volts with activated field position

4	Control lamps in the display unit

6	Switching position is discharge pipe moved out (opens when discharge pipe is moved in)

7	Connector at K 33 is currently not used (free)

- On 12 volts of the working lighting

Also see the circuit diagrams Lighting 4-401 A

Header Return

Circuit diagrams: 4-424 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Diodes | D 3 | Diode |  | Central processing unit |
|  | D 6 | Spark extinguishing diode |  | In front of the battery box |
| Trans-mitters | G 15 | Speed, upper shaft spindle | Initiator | Upper shaft spindle, right |
| Motors | M 9 | Header return | Starter motor | Shaft, front left |
| Relays | K 44 | Speed control, upper shaft spindle |  | Central processing unit |
|  | K 43 | Header rapid stop |  | Central processing unit |
|  | K A6 | Header return | Power relay | In front of the battery box |
| Switches | S 13 | Header return | Press for at least 5 seconds | Control panel |
|  | S 10 | Threshing unit clutch |  | Control panel |
|  | S 11 | Header clutch |  | Control panel |
|  | S 20 | Rapid stop | Header | On the joystick |
|  | S 80 | Master battery switch | 2-pole | In front of the battery box |
| Plugs | X 6 |  |  |  |
|  | X 8 |  |  |  |
|  | X 9 |  |  |  |
|  | X 10 |  |  |  |
|  | X 13 |  |  |  |
|  | X 16 |  |  |  |
|  | X 27 |  |  |  |
|  | X 45 | Plug, electronics subassembly, header return |  | On the electronics subassembly in the central processing unit |
|  | X 101 |  |  |  |

1	See the circuit diagrams Control System LH AGRO 4-431 A

- Pin 8 →   N upper shaft spindle (speed pulses of the upper shaft spindle)

Pin 18→  E upper shaft spindle (switch-in control of the upper shaft spindle)

3	Card subassembly Header Return

4	Switch-in conditions for the header return

● Field position is activated

● Threshing Unit Switch S10 is activated (threshing unit is switched in)

● Header Switch S11 is not activated (header is switched off)

● Standstill of the upper shaft spindle (speed control through G15)

● Hold down Header Return Switch S 13 for about 5 seconds

5	24 volts with activated field position

6	The connector for the harness is in the center on harness, on the left-hand side

7	On Relay K 42

8	Also see the circuit diagrams Threshing Unit and Header Clutch 4-422A

9	When threshing unit is switched in

Field Tracking Design Q (Electrical-Hydraulic Field Tracking)

Circuit diagrams: 4-426 B  from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Trans-mitters | G 22 | Shaft height |  |  |
|  | G 24 | Cross pendulum |  |  |
|  | G 25 | Pressure sensor |  |  |
|  | G 26 | Header probe skid | Outer right |  |
|  | G 27 | Header probe skid | Inner right |  |
|  | G 28 | Header probe skid | Inner left |  |
|  | G 29 | Header probe skid | Outer left |  |
| Solenoid valves | Y 7.1 | Lift shaft |  |  |
|  | Y 7.2 | Lower shaft |  |  |
|  | Y 8.1 | Cross-tracking, left |  |  |
|  | Y 8.2 | Cross-tracking, right |  |  |
|  | Y 9 | Supplemental valve |  |  |
| Relays | K 35 | Voltage supply transmitter |  | Central processing unit |
| Switches | S 24 | Lift / lower shaft |  | In the joystick, back |
|  | S 26 | Automatic field tracking. |  | In the joystick, front |
|  | S 37 | Boot switch |  |  |
| Plugs | X 7 |  |  |  |
|  | X 8 |  |  |  |
|  | X 9 |  |  |  |
|  | X 11 |  |  |  |
|  | X 13 |  |  |  |
|  | X 16 |  |  |  |
|  | X 17 |  |  |  |
|  | X 27 |  |  |  |
|  | X S | Header plug |  |  |

1	24 volts with field position "ON"

2	12 volts with ignition "ON"

3	24 volts with threshing unit clutch "ON"

4	Ground, central processing unit

5	Ground, machine frame

6	CCU control unit

7	DIL Switches 1 and 6 in "On" position

8	Operating and display device

9	Supplemental equipment

10	Joystick

11		      Shaft

Lift   Lower  Left  Right

- Further to reel switch in the joystick

Also see the circuit diagrams Working Hydraulics 4-430

Straw Cutter

Circuit diagram: 4-427 A starting with Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Trans-mitters | G 6 | Chopper speed | Speed initiator | Chopper shaft, left |
| Lamps | H SRA | Chopper control lamp |  | In the operating device |
|  | H SRA1 | Control lamp black | Not visible ( 5 ) | In the operating device |
| Solenoid valve | Y 14 | Chopper clutch |  |  |
| Motors | M 17 | Diffusing screen adjustment |  |  |
| Relays | K 15 |  |  |  |
|  | K 16 |  |  |  |
| Switches | S 76 | Straw guide plate | Limit stop switch | Straw flap, left |
| Plugs | X 6 |  |  |  |
|  | X 10 |  |  |  |
|  | X 16 |  |  |  |

1	Connector harness left

2	Connector harness center

3	Computer LH AGRO 968

4	24 volts with field position

5	Covered control lamp next to

- On threshing unit clutch switch

24 volts are applied when the threshing unit clutch is switched in

Rape Electrical System

Circuit diagrams:  4-428 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Motors | M 10 | Vertical sickle | 24 volt; right | Right header side wall |
|  | M 11 | Vertical sickle | 24 volt; left | Left header side wall |
| Relays | K 6 | Safety relay | Rape separator | Central processing unit |
|  | K 7 | Rape separator, right |  |  |
|  | K 8 | Rape separator, left |  | Central processing unit |
| Switches | S 12 | Rape separator | Switching Position l: M7 Switching Position ll:M7 and M8 |  |
| Plugs | X 7 |  |  |  |
|  | X 10 |  |  |  |
|  | X 80 | Plug of the rape separator | Right | Right header side wall |
|  | X 81 | Plug of the rape separator | Left | Left header side wall |
|  | X S | Header plug |  | On the header |
|  | X L | Socket of the rape separator | Left | Left header side wall |
|  | X R | Socket of the rape separator | Right | Right header side wall |

- 24 volts when the header clutch is switched in

Also see the circuit diagrams Threshing Unit and Header Clutches 4-422 A

Working Hydraulics Design Q (Electrical-Hydraulic Field Tracking)

Circuit diagrams: 4-430 A  from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |  |
| --- | --- | --- | --- | --- | --- |
| Solenoid valves | Y 1 | Reel speed | Proportional valve |  |  |
|  | Y 2.1 | Unloader pipe in |  |  |  |
|  | Y 2.2 | Unloader pipe out |  |  |  |
|  | Y 5.1 | Reel adjustment | Back |  |  |
|  | Y 5.2 | Reel adjustment | Forward |  |  |
|  | Y 6.1 | Reel adjustment | Lift |  |  |
|  | Y 6.2 | Reel adjustment | Lower |  |  |
|  | Y 7.1 | Lower shaft |  |  |  |
|  | Y 7.2 | Lift shaft |  |  |  |
|  | Y 8.1 | Cross-tracking | Left |  |  |
|  | Y 8.2 | Cross-tracking | Right |  |  |
|  | Y 9 | Supplemental valve |  |  |  |
| Relays | K 20 | Lower shaft | Not connected | Central processing unit |  |
|  | K 21 | Lift shaft |  |  |  |
|  | K 22 | Lift reel |  |  |  |
|  | K 23 | Lower reel |  |  |  |
|  | K 24 | Reel forwards |  |  |  |
|  | K 25 | Reel backwards |  |  |  |
|  | K 30 | Swivel out unloader pipe |  |  |  |
|  | K 31 | Swivel in unloader pipe |  |  |  |
| Switches | S 14 | Header latch |  | In the control panel |  |
|  | S 21 | Swivel discharge pipe | Out / in | In the joystick |  |
|  | S 24.1 | Shaft cross pendulum | Left right |  |  |
|  | S 24.2 | Lift / lower shaft |  |  |  |
|  | S 25.1 | Reel vertical adjustment | Lift / lower |  |  |
|  | S 25.2 | Reel horizontal adjustment | Forwards / backwards |  |  |
|  | S 26 | Switching in the automatic system of field tracking |  |  | In the joystick; front |
|  | S 51 | Pressure relief |  |  |  |
| Plugs | X 7 |  |  |  |  |
|  | X 8 |  |  |  |  |
|  | X 9 |  |  |  |  |
|  | X 11 |  |  |  |  |
|  | X 12 |  |  |  |  |
|  | X 13 |  |  |  |  |
|  | X 16 |  |  |  |  |
|  | X 17 |  |  |  |  |

1	Proportional amplifier, reel and fan speed

2	24 volts with field position

3		Shaft

Left  Right  Lift  Lower

4	Joystick

- Free; not used

- Control unit, shaft and basket adjustment (shaft function not used)

LH AGRO 968 Control System

Circuit diagrams: 4-431 A from Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Trans-mitters | G 1 | Threshing drum speed | Speed initiator | Threshing drum, left |
|  | G 2 | Fan speed | Speed initiator | Fan shaft, left |
|  | G 3 | Straw walker speed | Speed initiator | Straw walker shaft, left |
|  | G 4 | Engine speed | Speed initiator |  |
|  | G 5 | Driving speed | Speed initiator | Left rear wheel |
|  | G 6 | Chaff cutter | Speed initiator | Chopper shaft, left |
|  | G 7 | Loss transmitter (plate) | Straw walker | Left straw walker |
|  | G 8 | Loss transmitter (plate) | Cleaning unit | Cleaning unit, left |
|  | G 9 | Loss transmitter (plate) | Cleaning unit | Cleaning unit, right |
|  | G 10 | Loss transmitter (plate) | Straw walker | Right straw walker |
|  | G 11 | Loss transmitter (pipe) | For tailings | Under tailings auger in front of the threshing drum |
|  | G 12 | Speed, 4th drum | Speed initiator | Shaft, 4th drum, right |
|  | G 13 | Grain elevator speed | Speed initiator | Grain elevator, right |
|  | G 14 | Tailings elevator speed | Speed initiator | Tailings elevator, right |
|  | G 15 | Shaft speed | Speed initiator | Upper shaft spindle, right |
|  | G 16 | Harvested grain switch | Speed initiator | Shaft inlet, right |
| Relays | K 17 | Adjustment, fan speed |  | Central processing unit |
|  | K 18 | Adjustment, threshing drum speed |  | Central processing unit |
| Plugs | X 2 |  |  |  |
|  | X 6 |  |  |  |
|  | X 6.1 |  |  |  |
|  | X 8 |  |  |  |
|  | X 9 |  |  |  |
|  | X 16 |  |  |  |
|  | X 27 |  |  |  |
|  | X 101 | Computer plug |  | Cab roof, right front |

1	Free; not used

2	Computer LH AGRO 968

3	Signal decoupling for fan and reel speed

4	12 volts with ignition "On"

5	Switching-in conditions, chaff cutter; also see the circuit diagrams 4-427

6	Chaff cutter connector near the battery box

7	Connection for extension cable, harness on the right

8	White connection housing of the loss measurement in the straw area hood, right

Electrical Central Lubrication System

Circuit diagrams: 4-440 A starting with Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Motors | M 18 | Pump, centralized lubrication |  | Right side wall |
| Relays | K 58 | Switching-on conditions, centralized lubrication |  | Central processing unit |
| Plugs | X 2 |  |  |  |
|  | X 9 |  |  |  |
|  | X 13 |  |  |  |
|  | X 16 |  |  |  |

- 12 volts with ignition on

Also see the circuit diagrams Operating Voltage System 4-400

2	24 volts with threshing unit clutch "On"

3	Ground, central processing unit

4	Ground, machine frame

5	Free, not used

6	Control lamps (Positions 25 and 26 ) in the operating device

7	Contacts (2) for displaying an operational malfunction

All-Wheel Axle

Circuit diagrams:  4-441 A starting with Machine Number DCF 860103

Circuit Diagram Legend

|  | Number | Function | Remark | Location |
| --- | --- | --- | --- | --- |
| Solenoid valves | Y 15 | All-wheel | 12 volt |  |
| Switches | S 27 | All-wheel |  | Operating device |
| Plugs | X 2 |  |  |  |
|  | X 9 |  |  |  |
|  | X10 |  |  |  |
|  | X 13 |  |  |  |
|  | X 30 |  |  |  |

- 12 volts when ignition "On"

- Ground, central processing unit

- Ground, machine frame

- Connection in cable, steering axle support